Polycom Provisioning

The purpose of this document is to detail out the process of factory resetting and provisioning a Polycom device. When transitioning a company to our phone system, the phone will need to be factory reset before being provisioned to our server.

Factory Resetting Polycom Device

- Turn off phone

- Power phone back up and wait for the Welcome screen.

- As soon as Welcome displays, press and hold down

  - “1” “3” “5” “7” (on 320, 321, 330, 331, 450, 560)

  - “4” “6” “8” “*” (on 301, 501, 550, 600, 601, 650, 670)

- Only release the combination once the phone prompts you for the password

  - Password should be the MAC address of the device

The phone will reboot after the password

- Locate the IP address of the phone by doing the following:

- Press Menu  Status  Network  TCP/IP Parameters, the IP address will be prefixed with IP:

- Navigate to the IP address using your preferred web browser.

- Default password should be 456. (If previously provisioned by LOOP, password = 222222)

- Once logged in, mouse over Settings and click Provisioning Server

- Select TFTP for Server Type

- Input server IP address for Server Address

- Click SAVE

The phone will automatically reboot.